SECTION 04 20 00

COLORED CONCRETE MASONRY UNITS

Davis Colors add new dimensions to masonry design. Our permanent, integral color additives are made with pure, concentrated pigments specially treated for mixing into concrete products and mortar. They are lightfast, alkali-resistant, weather-resistant, and formulated to give long-lasting appeal to masonry.

Colored concrete masonry unit (CMU) walls do not have to be painted, nor do they require periodic refinishing as can coatings and applied finishes. True Tone cement colors can be used for mortar colors that match or compliment masonry units to create a variety of effects.

As with all masonry products, comply with American Concrete Institute (ACI), National Concrete Masonry Association (NCMA) and Brick Industry Association (BIA) recommendations and other applicable industry guidelines.

Specification Coordination: Insert clauses from this document into a complete section specifying masonry construction.

This document is available in word processing format at www.daviscolors.com/Literature. Turn on HIDDEN TEXT to view or print specifier notes.

Drawing Coordination: Show locations of colored concrete masonry units on Drawings or in Schedules.

Samples and Technical Assistance: Davis Colors

Phone: 800-356-4848 (toll free) or 323-269-7311.

E-mail: info@daviscolors.com. Internet: www.daviscolors.com.

- GENERAL

      - SUMMARY

        - Section Includes:

          - Colored concrete masonry units (CMU).

          - [Colored mortar.]

        - Related Requirements:

          - Division 07 Section ["Joint Sealants"]<Other title>: Colored sealants for joints.

      - REFERENCE STANDARDS

Edit references to include standards used in Project Specifications.

The following useful publications are available at www.daviscolors.com/literature:

"Manufacturing Colored Concrete Masonry", Concrete Products, October 1995.

Portland Cement Association Publication IS247.01M – “Trowel Tips: Mortar Color.”

        - ACI 530 - Building Code Requirements for Masonry Structures.

        - ACI - 530.1 - Specifications for Masonry Structures.

        - ASTM C55 - Concrete Brick.

        - ASTM C90 - Loadbearing Concrete Masonry Units.

        - ASTM C270 - Mortar for Unit Masonry.

        - ASTM C979 - Pigments for Integrally Colored Concrete.

      - SUBMITTALS

        - Product Data:

          - Mortar colors.

          - Proprietary cleaning agents.

          - <Other Product Type>

For LEED projects, include submittal requirements for targeted credits. Davis Colors support LEED Version 2009 credits listed below, and may assist in attaining additional credits.

Recycled Content: Many True Tone colors are made with preconsumer reclaimed and recycled metal. Recycled content is 60%.

Regional Materials:  Davis Colors has manufacturing facilities in Beltsville, MD; King of Prussia, PA; Los Angeles, CA; Europe; and Asia. Raw materials are extracted from domestic and international sources.

        - LEED Submittals: Submit data for:

          - Recycled Content: LEED Credit MR 4.1 [and MR 4.2] <Other credit>.

          - Regional Materials: LEED Credit MR 5.1 [and MR 5.2] <Other credit>.

        - Samples:

Coordinate CMU sample requirements with method used to specify CMU color in Part 2.

          - Colored CMU:

Use first method when colors have already been selected from CMU manufacturer’s standards.

            - Submit samples of [each color and style of] specified CMU indicating color [range and] appearance.

Use second method when colors will be selected from CMU manufacturer’s standards.

            - Submit samples of CMU manufacturer's standard colors for selection.

Use third method when colors are to match an existing building or sample.

            - Prior to manufacturing CMU, submit laboratory samples of colored concrete for preliminary color matching. Following preliminary approval, submit samples of [each color and style of] CMU taken from an actual production run.

Use fourth method when Davis Colors are used as a color standard to specify CMU color.

            - Submit color additive manufacturer's [color chart] [sample chip set] for color selection. Submittals are for general color selection and may vary somewhat from CMU manufactured at plant. Following preliminary selection, submit samples of [each color and style of] CMU taken from an actual production run.

Use fifth method when concrete mix ingredients are specified.

            - Prior to manufacturing CMU, submit laboratory samples of colored concrete for preliminary color matching. Following preliminary approval, submit samples of [each color and style of] CMU taken from an actual production run.

          - Colored Mortar:

            - Submit color manufacturer's [color chart] [sample mortar channel set] for color selection.

Include above if color selection has not yet been made. Include below if color selection is specified.

            - Submit [swatch[es] from color chart] [hardened mortar sample[s]] of specified color[s] indicating color number[s] and dosage rate[s].

            - Submittals are for general indication of color and may vary somewhat from mortar finished in field.

        - Certification:

          - Colored CMU: Submit product data or letter of certification from CMU manufacturer stating color additives comply with specifications.

Coordinate "Qualification Data" Paragraph below with qualification requirements in "Quality Assurance" Article.

        - Qualification Data: For Manufacturer and Installer.

      - QUALITY ASSURANCE

        - Perform work in accordance with ACI 530 and ACI 530.1.

        - Obtain each material from same source and maintain high degree of consistency in workmanship throughout Project. Assure that enough inventory of each material is available to meet Project requirements and contingencies.

        - Manufacturer Qualifications: Concrete masonry manufacturer shall have minimum 5 years experience manufacturing colored concrete masonry and an acceptable quality control program.

        - Installer Qualifications: Masonry work shall be by firm with [five]<Other Number> years experience with work of similar scope and quality.

On jobs with critical appearance tolerances, specify a mock-up to demonstrate that proposed materials and workmanship produce acceptable masonry appearance.

        - Mock-Up:

          - Provide mock-up of wall made with colored concrete masonry units.

          - Provide mock-up under provisions of Division 01 Section ["Quality Requirements"]<Other Section Title>. Construct one month before start of other masonry work to allow mortar to cure before observation.

          - At location selected by [Architect]<Other Party>, demonstrate methods used for construction, including colors required for Work, using materials, workmanship, joint treatments, and cleaning techniques to be used throughout Project.

          - Accepted mock-up provides visual standard for work of Section.

If allowing mock-up to remain as part of Work, locate it where any inconsistency of color or workmanship will be inconspicuous.

          - [Mock-up may remain as part of Work, if undisturbed at time of Substantial Completion]. [Remove mock-up when no longer required for comparison with finished work.]

      - DELIVERY, STORAGE, AND HANDLING

        - Mortar Colors: Deliver color additives in original, unopened packaging. Store in dry conditions. Comply with manufacturer's instructions.

- PRODUCTS

      - CONCRETE MASONRY UNITS

In general, integrally colored concrete masonry units can be specified with same requirements used for high-quality uncolored CMU. Colors of cement, supplementary cementitious materials, and coarse and fine aggregates affect appearance of colored concrete.

        - Manufacturers:

          - [Name manufacturer].

          - [Name manufacturer].

        - Substitutions: Comply with provisions of Division 01 Section ["Substitution Procedures"]<Other Section Title> for substitution request procedures.

        - Type: [ASTM C90, Type [I] [II]; [normal] [medium] [light] weight.] [ASTM C 55.] <Other standard.>

        - Size: [Nominal modular size shown in Drawings]<Other size>.

        - Face: [Smooth] [Scored] [Ribbed] [Split] [Ground] [Slump] [Striated] <Other requirement>.

        - Admixtures: Do not use calcium chloride admixtures.

In general, select colors from CMU manufacturer’s standard colors or, if required, specify custom color.

Select one of following color specification methods to coordinate with method of color selection and sample submittal requirements.

Use first method if colors have already been selected from CMU manufacturer's standards.

        - CMU Color[s]:

          - [CMU manufacturer name]; <Add color number or name>.

Use second method if colors will be selected from CMU manufacturer’s standards.

          - CMU Color[s]: Provide color[s] to be selected by [Architect]<Other Party> from CMU manufacturer’s [standard] [premium] color line. [Allow for up to [two]<number> colors on Project.]

Use third method if colors are to match an existing building or sample.

          - CMU Color[s]: Match [existing building] [Architect’s sample]<Other item to match>.

Use fourth method to specify Davis Colors as a color standard for custom CMU colors. The Davis Color Selector groups colors into categories to allow more accurate cost estimating and bidding; subtle=$, standard=$$, premium=$$$, ultra-premium=$$$$, and elite=$$$$$.

          - CMU Color[s]: Provide CMU with color additives from Davis Colors. [Allow for up to [two]<number> colors on Project.]

            - Match concrete colors selected by [Architect]<Other Party> from Davis Colors’ [subtle] [standard] [premium] [ultra-premium] [and] [elite] color line[s].

            - Manufacturer: Davis Colors

              - Phone: 800-356-4848.

              - E-mail: info@daviscolors.com

              - Web Site: www.daviscolors.com

Use fifth method when specifier requires greater control of mix design.

Colors can be selected from Davis Colors color cards, samples, or Concrete Palette. Concrete Palette, available at www.concretepalette.com, is color-visualizing software that allows you to create custom concrete colors and order samples. Colors selected using Concrete Palette are assigned unique numbers by software; descriptive names can be given by project designer or specifier.

          - CMU Color[s]: Provide CMU with color additives from Davis Colors.

            - [CMU Color]<Other designation>: Davis Colors No. <Add color number and/or color name>.

            - Manufacturer: Davis Colors

              - Phone: 800-356-4848.

              - E-mail: info@daviscolors.com

              - Web Site: www.daviscolors.com

            - Color additives shall contain pure, concentrated pigments specially processed for mixing into concrete and complying with ASTM C979.

Pigments containing carbon blacks should not be used in concrete masonry units. Instead, black iron oxide pigments (Davis Colors #860, #77, or Onyx) should be used to create black or gray concrete masonry units.

            - Color additives containing carbon black are not acceptable.

White cement produces a lighter color CMU.

            - Cement Color: [Gray.] [White.]

Specify sand and aggregates colors when using split face and ground units.

            - Aggregate: [Match Architect’s sample.]<Insert other requirement.>

      - COLORED MORTAR

Specify colored mortar as you would uncolored mortar. Cement and sand colors affect appearance of mortar.

        - Cement Color: [Gray.] [White.]

        - Sand Color: [Locally available natural sand.] [White.]

True Tone cement colors are available in 16 standard colors that can be mixed to produce 64 different hues. Dose-sized boxes reduce problems associated with weighing colors in field. Use white cement to obtain lightest colors on True Tone color card.

        - Mortar Colors:

          - Product: Davis Colors, True Tone Cement Colors.

            - Phone: 800-356-4848.

            - E-mail: info@daviscolors.com

            - Web Site: www.daviscolors.com

          - Substitutions: [Comply with [Instructions to Bidders] [Division 01 Section “Product Requirements”]<Other requirement> for substitution request procedures.] [Not Allowed.]

          - Materials: Mortar colors shall contain pure, concentrated mineral pigments especially processed for mixing into mortar and complying with ASTM C979.

          - Color: Provide color to be selected by [Architect]<Other Party> from manufacturer's [light] [medium] [or] [dark] Select Series colors.

Include subparagraph above if color selection has not yet been made. Include one of following subparagraphs if color selection is specified.

          - Color: Provide True Tone Color No.  [_______] at dose rate of [_______] lbs per sack of cement or lime.

          - Color: Match [existing building.] [Architect’s sample.]<Insert other sample to match.>

      - ACCESSORIES

        - Joint Sealants: Provide [type specified in Division 07 Section "Joint Sealants".] <Other Requirements>. Sealant shall [be color selected by [Architect]<Other Party> from manufacturer's full range to] match colored masonry.

        - Cleaning Agents: Use products known to be compatible with colored concrete and mortar.

      - MORTAR MIXES

Specify mortar mix according to architectural and structural requirements.

Variations in water-cement ratio affect appearance of mortar. Low water-cement ratio promotes richer, darker mortar colors.

        - Mortar Color: Mix in accordance with manufacturer's instructions.

          - Thoroughly mix using mechanical mixer.

          - Sand shall be uniformly damp immediately before mixing.

          - Mix until color additives are uniformly dispersed throughout mixture.

        - Provide a consistent water-cement ratio. Do not retemper or add additional water to colored mortar.

      - SCHEDULE

Modify schedules to reflect project requirements. Information in schedule is for example only.

| Location | CMU Color | CMU Finish | True Tone Mortar Color |
| --- | --- | --- | --- |
| Exterior Field Units | Davis Colors Sierra | Split Face | 160 Medium |
| Exterior Accent Units | Davis Colors Brick Red | Split Face | 160 Medium |
| Interior Units | Davis Colors Sierra | Smooth Face | 160 Medium up to 48” above finish floor; natural color above 48”. |

- EXECUTION

      - INSTALLATION

Specify installation of colored CMU in much the same manner as uncolored masonry and mortar. Observe good masonry practices.

        - Use careful workmanship to minimize mortar splatters and smears on face of masonry.

        - Colored Mortar: Tool mortar joints at same degree of hardness and moisture content.

        - Cover tops of walls to prevent water penetration into wall during construction.

      - TOLERANCES

As with any natural material, some variation in appearance is a normal design feature of masonry, whether colored or not. It is normal for color of mortar to lighten as it cures; allow up to 28 days for process to occur.

        - Minor variations in appearance of colored masonry units and mortar that are similar to natural variations in appearance of uncolored masonry work are acceptable.

      - CLEANING

        - Use least aggressive cleaning techniques possible. Verify cleaning procedure is consistent with recommendations of masonry unit and color manufacturers.

Washing a wall while mortar is still fresh can smear colors and cement across face of masonry units and make cleaning more difficult.

        - Excess Mortar:

          - Let mortar splatters harden for 7 to 14 days, then remove with chisel, trowel, or stiff brush and water before splatters bond rigidly to masonry.

          - Prior to further cleaning, allow masonry to continue curing for at least three weeks in summer or four weeks in winter.

        - If proprietary cleaning agents are used, pre-wet wall, test cleaning agent on [mock-up] [a small, inconspicuous area], and check effects prior to proceeding. Begin cleaning at top and work down. Thoroughly rinse wall afterwards with clean water. Follow cleaner manufacturer's instructions.

        - Do not use muriatic (hydrochloric) acid on colored CMU.

Efflorescence occurs when soluble salts in concrete are carried to surface where they are deposited by evaporation. Since powdery efflorescence deposits are white, they are more visible on dark colored masonry than on light, uncolored concrete masonry. If not removed while still fresh, efflorescence converts to calcium carbonate deposits, which are permanent.

        - Efflorescence: Remove efflorescence [as soon as practical after it appears] [and] [as part of final cleaning of building].

END OF SECTION

Davis Colors®, Concrete Palette™, Select Series™ and True Tone™ are trademarks of Davis Colors/Rockwood Pigments NA, Inc. Copyright 2012 by Davis Colors/Rockwood Pigments NA, Inc. Qualified design and construction professionals may copy for preparation of construction specifications. Issued March 2012.