SECTION 04 05 13

COLORS FOR MASONRY MORTARING

True Tone cement colors, from Davis Colors, are made with pure, concentrated pigments specially treated for mixing into mortar. They are lightfast, alkali-resistant, weather-resistant, and formulated to give long-lasting appeal to masonry.

Mortar joints comprise up to 20 percent of a masonry wall’s surface, so color has a big impact on masonry appearance. Mortar colors can match or compliment masonry units to create a variety of effects.

Specification Coordination: Insert clauses from this document into a complete section specifying masonry construction.

This document is available in word processing format at www.daviscolors.com/Literature. Turn on HIDDEN TEXT to view or print specifier notes.

Drawing Coordination: Show locations colored mortar on Drawings or in Schedules.

Samples and Technical Assistance: Davis Colors

Phone: 800-356-4848 (toll free) or 323-269-7311.

E-mail: info@daviscolors.com. Internet: www.daviscolors.com.

- GENERAL

      - SUMMARY

        - Section Includes:

          - Colored mortar.

        - Related Requirements:

          - Division 07 Section ["Joint Sealants"]<Other title>: Colored sealants for joints.

      - REFERENCE STANDARDS

Edit references to include standards used in Project Specifications.

Portland Cement Association Publication IS247.01M – “Trowel Tips: Mortar Color” is available at www.daviscolors.com/literature.

        - ASTM C270 - Mortar for Unit Masonry.

        - ASTM C979 - Pigments for Integrally Colored Concrete.

      - SUBMITTALS

        - Product Data:

          - Mortar colors.

          - Proprietary cleaning agents.

          - <Other Product Type>

For LEED projects, include submittal requirements for targeted credits. True Tone cement colors support LEED Version 2009 credits listed below, and may assist in attaining additional credits.

Recycled Content: Many True Tone colors are made with preconsumer reclaimed and recycled metal. Recycled content is 60%.

Regional Materials:  Davis Colors has manufacturing facilities in Beltsville, MD; King of Prussia, PA; Los Angeles, CA; Europe and Asia. Raw materials are extracted from domestic and international sources.

        - LEED Submittals: Submit data for:

          - Recycled Content: LEED Credit MR 4.1 [and MR 4.2] <Other credit>.

          - Regional Materials: LEED Credit MR 5.1 [and MR 5.2] <Other credit>.

        - Samples:

          - Colored Mortar:

            - Submit color manufacturer's [color chart] [sample mortar channel set] for color selection.

Include above if color selection has not yet been made. Include below if color selection is specified.

            - Submit [swatch[es] from color chart] [hardened mortar sample[s]] of specified color[s] indicating color number[s] and dosage rate[s].

            - Submittals are for general indication of color and may vary somewhat from mortar finished in field.

      - QUALITY ASSURANCE

        - Obtain each material from same source and maintain high degree of consistency in workmanship throughout Project.

If project requires a mock-up, specify that it include colored mortar.

        - Mock-Up: Construct mock-up one month before start of other masonry work to allow mortar to cure before observation. Demonstrate materials, workmanship, joint treatments, and cleaning techniques to be used with colored mortar.

      - DELIVERY, STORAGE, AND HANDLING

        - Mortar Colors: Deliver color additives in original, unopened packaging. Store in dry conditions. Comply with manufacturer's instructions.

- PRODUCTS

      - COLORED MORTAR

Specify colored mortar as you would uncolored mortar. Cement and sand colors affect appearance of mortar.

        - Cement Color: [Gray.] [White.]

        - Sand Color: [Locally available natural sand.] [White.]

True Tone cement colors are available in two formulations. True Tone ‘Sweet 16’ are 16 reduced strength colors for premixed mortar that can be mixed to produce 64 different hues. True Tone ‘Select Series’ are full-strength colors for large mortar batches and can also be mixed to produce 64 different hues. All True Tone cement colors can be used in any mortar mix, concrete or product made with cement. Most colors are made with gray cement, but the lightest colors require the use of white cement. All True Tone cement colors are packages in dose-sized boxes to reduce problems associated with weighing colors in the field.

        - Mortar Colors:

          - Product: Davis Colors, True Tone [Sweet 16] [Select Series] Cement Colors.

            - Phone: 800-356-4848.

            - E-mail: info@daviscolors.com

            - Web Site: www.daviscolors.com.

          - Substitutions: [Comply with [Instructions to Bidders] [Division 01 Section “Product Requirements”]<Other requirement> for substitution request procedures.] [Not Allowed.]

          - Materials: Mortar colors shall contain pure, concentrated mineral pigments especially processed for mixing into mortar and complying with ASTM C979.

          - Color: Provide color to be selected by [Architect]<Other Party> from manufacturer's standard colors.

Include subparagraph above if color selection has not yet been made. Include one of following subparagraphs if color selection is specified.

          - Color: Provide True Tone [Color No.]  at a dose rate of [___Lbs] per sack of cement or lime.

          - Color: Match [existing building.] [[Architect’s]<Other Party’s> sample.]<Insert other sample to match.>

      - ACCESSORIES

        - Joint Sealants: Provide [type specified in Division 07 Section "Joint Sealants"]<Other Requirements>. Sealant shall [be color selected by [Architect]<Other Party> from manufacturer's full range to] match mortar color.

        - Cleaning Agents: Use products known to be compatible with colored mortar.

      - MORTAR MIXES

Specify mortar mix according to architectural and structural requirements.

Variations in water-cement ratio affect appearance of mortar. Low water-cement ratio promotes richer, darker mortar colors.

        - Mortar Color: Mix in accordance with manufacturer's instructions.

          - Thoroughly mix using mechanical mixer.

          - Sand shall be uniformly damp immediately before mixing.

          - Mix until color additives are uniformly dispersed throughout mixture.

        - Provide a consistent water-cement ratio. Do not retemper or add additional water to colored mortar.

- EXECUTION

      - INSTALLATION

Specify installation of colored mortar in much the same manner as uncolored mortar. Observe good masonry practices.

        - Use careful workmanship to minimize mortar splatters and smears on face of masonry.

        - Colored Mortar: Tool mortar joints at same degree of hardness and moisture content.

        - Cover tops of walls to prevent water penetration into wall during construction.

      - TOLERANCES

As with any natural material, some variation in appearance is a normal design feature of masonry, whether colored or not. It is normal for color of mortar to lighten as it cures; allow up to 28 days for process to occur.

        - Minor variations in appearance of colored mortar that are similar to natural variations in appearance of uncolored mortar are acceptable.

      - CLEANING

        - Use least aggressive cleaning techniques possible. Verify cleaning procedure is consistent with recommendations of masonry unit and color manufacturers.

Washing a wall while mortar is still fresh can smear colors and cement across face of masonry units and make cleaning more difficult.

        - Excess Colored Mortar:

          - Let colored mortar splatters harden for 7 to 14 days, then remove with chisel, trowel, or stiff brush and water before splatters bond rigidly to masonry.

          - Prior to further cleaning, allow masonry to continue curing for at least three weeks in summer or four weeks in winter.

        - If proprietary cleaning agents are used, pre-wet wall, test cleaning agent on [mock-up] [a small, inconspicuous area], and check effects prior to proceeding. Begin cleaning at top and work down. Thoroughly rinse wall afterwards with clean water. Follow cleaner manufacturer's instructions.

        - Do not use muriatic (hydrochloric) acid on colored mortar.

Efflorescence occurs when soluble salts in concrete are carried to surface where they are deposited by evaporation. Since powdery efflorescence deposits are white, they are more visible on dark colored masonry than on light, uncolored concrete masonry. If not removed while still fresh, efflorescence converts to calcium carbonate deposits, which are permanent.

        - Efflorescence: Remove efflorescence [as soon as practical after it appears] [and] [as part of final cleaning of building].

END OF SECTION

Davis Colors®, Sweet 16™, Select Series™ and True Tone™ are trademarks of Davis Colors/Rockwood Pigments NA, Inc. Copyright 2012 by Davis Colors/Rockwood Pigments NA, Inc. Qualified design and construction professionals may copy for preparation of construction specifications. Issued March 2012.